BUILDING CONSTRUCTION AGREEMENT – GROUND FLOOR

- FOUNDATION

1’0”x 9” COLUMN – 3’6”LENGTH X 3’6” FEET WIDTH X 5’ FEET DEPTH

2 .  P.C.C. CONCRETE & P.C.C 1:4:8  RATIO- 4”HEIGHT-COLUMN PIT.

FOR FLOORING CONCERTE 3” THICK.

3.	FOOTING CONCRETE : 1’0”X 9”COLUMN  -  3’6” X 3’6” X 1’9”

- COLUMN CONCRETE :

- 1’0” X0’9” COLUMN SIZE–( 4 NOS -16MM & 2Nos – 12mm, DIA- MAIN ROD & RINGS – 8MM DIA, 6”C/C) – (upto Roof beam level)

- PLINTH BEAM  :

- 10’0” SPAN–BEAM SIZE-0’9” X 1’0” (5 Nos. – 12mm Dia Rod)

- 16’0” SPAN- BEAM SIZE -0’9”X 1’3” (5 Nos. – 16mm Dia)

- BASEMENT LEVEL	 : Existing road level to FFL level -  3’0” Height.

- BRICK WORK:     10’0” Feet height clear size cement Mortar - 1:6 ratio

- LINTEL CONCRETE:    7’0” Feet height  Lintel,  size - 0’9” x 0’6” height ( Steel - Main rod – 10 mm @  – 4 Nos     and Rings – 8 mm dia – 9”c/c

- ROOF  BEAM  :

- 10’0” SPAN–BEAM SIZE-0’9” X 1’0” (5 Nos. – 12mm Dia Rod)

- 16’0” SPAN- BEAM SIZE -0’9”X 1’3” (5 Nos. – 16mm Dia)

- ROOF CONCRETE:   5” Thickness concrete. slab main rod – 10mm dia – 6” c/c and distributor – 8 mm dia – 9”c/c (One way Slab)   6” Thickness concrete. slab main rod – 10mm dia – 6” c/c  Bothways (Two way Slab)   Concrete mix – 1:2:4

- PARAPET WALL :  2’6” height – 9” thick wall above roof slab

- WHEATERING COURSE:  Ellispetern Flooring Concerte with using 6mm Jelly

- KITCHEN ROOM: Table top concrete 2” thick with polished black Granite slab &   Stainless steel sink fitted. side wall tiles 7’0” height (tiles cost- 50.00/ sft)  Cuddapah shelves – 7’0” height x 3’0” width

- FLOORING WORK:  All rooms – Vetrified tiles, with 3” skirting tiles work for all side walls. (Basic rate of tiles – 50 / sft)

- BATH ROOM:   Flooring – Antiskid tiles, side wall 7’0” height concept tiles ( Basic rate tiles – 50 / sft)

- ELECTRICAL WORK:  Wire – Finolex brand & Switches – Modular switches.

- PLUMBINGWORK: Tank line and distribution line – 1” thick U-PVC pipe with necessary G.I.fittings. Wall mixer, Shower & Taps – ISI Brand – Water Tank 1000 Liters ( ISI Make)

- SANITARY WORK:  EWC, Wash basin and Indian closet all items ISI brand

- MAINDOOR:  7’0” feet height 5’0” width – Padak  wood door with polish work and side Mirror

- WINDOWS AND INNER DOOR:  Inner door frame – Sal wood and readymade

PUF – door.   (Rs.3000/-)  Windows – Sal wood frame and shutter with designed glass. Window grill – 12mm square rod with Ordinary Design.

- PAINTING WORK:

  - Inner wall surface – 2 coats of  Power wall care putty and one coat of primer.

  - Exterior wall surface – 2 coat of kottayam cement

NOTE:

- Cement 	- Chettinad, Priya

- Bricks		- Chamber bricks

- Sand		- Trichy Sand

- Jelly		- Kinathukadavu

- Water and E.B supply are free of cost @ site

- Elevation work and overhead water tank Extra cost ( or) Actual labour cost

- Compound wall with gate extra cost

BUILDING AREA DETAILS:

Area of Ground Floor Main Building- 1 sft x 1450.00                                  = 1450.00

Area of Ground Floor Portico & Staircase – 1 sft x 900.00			= 900.00

Area of First Floor Building 1 sft x 1400.00					=1400.00

Septic Tank	(Lumsum)-Size(10’x6’x9’)outer to outer			= 50000.00

Water Tank – 1 Ltr x 10.00(per litre) – Brick wall 			            = 10.00

Water Tank – 1 Lt x 15.00(per Litre) – Concrete Wall 			= 15.00

Compound Wall – 1 Rft x 1000.00 –Brick wall				=1000.00

Compound Wall – 1 Rft x 900.00 – Hollow Block 				= 900.00

Gate 200 kg(App.Wt.)	(10 feet gate + 5feet gate)				= 20,000.00

Outer Toilet (or) Staircase Below Toilet 					= 20,000.00

PAYMENT DETAILS:

ADVANCE			:

BASEMENT LEVEL		:

LINTEL LEVEL		:

ROOF LEVEL		:

PLASTERING LEVEL	:

INNER			:

OUTER			:

FINAL PAYMENT		:

TOTAL ESTIMATE Rs. 	:

CLIENT SIGNATURE						CIVIL ENGINEER SIGNATURE